	[image: image1.png]



Raul E. Sesin, P.E., CFM

PLANNING ADMINISTRATOR
	PLANNING DEPARTMENT

County Of Hidalgo


TO:
Judge Juan D. Salinas III

County Commissioner’s Court

FROM:
Raul E. Sesin, P.E., Planning Administrator

DATE:
May 2, 2007

RE:
Pueblo de Palmas Phase 7 Subdivision – Pct. 3

Preliminary Approval 
Pueblo de Palmas Phase 7 is a eight six (86) lot subdivision located approximately 3282 feet South of its intersection with Mile 7 Road (FM 2221) and Sabal Palm Drive.

The proposed Subdivision lies within the Rural Area of the County.

The Drainage Report was submitted to, reviewed, and approved by the H.C.D.D. No. 1 Manager on August 24, 2004.  The proposed subdivision lies within Zone “A & C” as per FEMA’s FIRM.

Sanitary Services will be provided by OSSF.  Soil Analysis were conducted by Isidoro Gomez and approved by the Hidalgo County Environmental Health Department.  

Water Services will be provided by La Joya Water Supply Corporation.  There is an existing eight (8) inch waterline on Sago Palm Drive, an existing eight (8) inch waterline on Sabal Palm Drive and an existing eight (8) inch on Coconut Palm Drive that will serve as the primary source of water for the proposed development.

The Subdivision Plat has been reviewed and complies with the Hidalgo County Subdivision Rules, Texas Water Development Board Model Subdivision Rules and the Texas Local Government Code.  Staff comments are attached for your review.

The Hidalgo County Advisory Board convened and recommended Preliminary Approval on May 1, 2007 subject to staff comments and future recommendations by Planning and other departments.
[image: image2.png]LOG OF ITEMS LACKING
DURING SUBDIVISION REVIEW

Name of Subdivision: Pueblo de Palmas Phs 7 1%Review Page  1of2

This form lists the documents lacking during the process of seeking subdivision approval. The County official
receiving a document should upon its receipt fill out and initial an entry for the items lacking and then write the
log number on the document. If needed use more than one line for the description.

Item Logger’s
Log | Description of Item Date Initials
#
A 1" SHEET COMMENTS: 04-26-07 JRT
PLAT NOTES:

#2) Front setback should be ' of the total road ROW not to exceed 50
feet. On a 100 foot road ROW the front setback is 50 feet.

#6) Correct acre feet conversion from total cubic feet detention.

-> Add a plat note for a special OSSF design for all lots with in a
designated flood zone. (see attachment sample)

B | METES & BOUNDS: «“ “

o 3" callout: provide correct recorded documentation for Pueblo de
Palmas Phase 6

e 12" and 13® callout: please mention the existing 150 foot drain
ditch easement.

C | INDEX TO SHEETS CHART: “ “
Correct by mention the fallowing for each sheet:
e sheet 1: mention the engineer certification
e Sheet 2: Subdivider certificate and statement
e Sheet 3: construction details.

D | Will irrigation district require approval for this subdivision? “ “

E 2" SHEET COMMENTS: < @
Provide south and south east adjoining property owners name and
document number.

F | Provide lot pins for lots 577-A & 577-B. Will these lots sale together or “ «“
separately? Why are they being label as “A” & “B™?

G | Documentation for HCDD#1 drainage ditch is for an easement not for a “ “
ROW, please correct labeling.

H | Provide volume and page recorded document for east adjoining Pueblo De « “
Palma Phs 6 subdivision.

I 3" SHEET COMMENTS: “ “
WATER ENGINEER REPORT:

Verify amount of % single services.

J | All total amount regarding 30 year letter, construction cost for water “ “
distribution, and letter of credit for OSSF should be provide prior to final
original subdivision plat.

K | All modification on engineer report need to reflect on Spanish version. «“ “

L | Provide notary public certification and acknowledgment block for “ “
subdivider certificate and statement.





[image: image3.png]LOG OF ITEMS LACKING
DURING SUBDIVISION REVIEW

Name of Subdivision: Pueblo de Palmas Phs 7 1%'Review

Page

2 0f2

M

Correct engineer certification on drainage report to mention the following:
1. Location and description of flood zone
2. community panel and revised date
3. Based on graphical representation on date sigh. (see attach sample)

04-26-07

JRT

N | Label all fire hydrant with flange elevation “ “
O | Label solid line located in the center line of Sago Palm. ¢ “
P | Provide OSSF for lot 577-B and 578-B. « “
Q 4" SHEET COMMENTS: “ “
Provide street grading elevation for all proposed streets.
R | Label and show all drainage swales and provide cross section detail. “ “
S | Provide top and flow elevation for all typ “CC” inlets. “ “
H | Platis subject to additional comments from Planning, Right of Way, “ “

Health, and HCDD No. 1 prior to consideration for final approval.





1304 S. 25th St. ( Edinburg, Texas 78539 ( (956) 318-2840 ( Fax. (956) 318-2844 www.hchd.org/planning


