


EXHIBIT “A”
HIDALGO COUNTY ADDITIONAL SCOPE OF SERVICES
“CABLING – INSTALLATION - TIME – MATERIALS – OTHER SERVICES”
I. DESCRIPTION  
Building structured wiring systems shall meet Hidalgo County Tele-Data Cabling Conventions to include adherence to the most currently available Building Industry Consulting Service International (BICSI) Telecommunications Distribution Methods Manual (TDMM 10th Edition), ANSI/TIA/EIA Telecommunications Building Wiring Standards, and National Electrical Code NFPA 70 & 72 manuals. Proposers shall be fully acquainted with the above referenced standards and be fully qualified, as outlined in the Hidalgo County Tele-Data communications contractor qualifications, to propose on and perform work. Proposer(s) shall have demonstrated qualifications to install and test Fast Ethernet and Gigabit Ethernet Copper & Fiber cabling. Additionally, the successful proposer will be required to meet Hidalgo County Tele-Data conventions and standards. The successful Proposer will be required to meet with and coordinate with a representative of the IT Networking and Telecommunications Department prior to work beginning, and weekly, during the installation process. Weekly meetings will include a site inspection to ensure compliance with the before mentioned standards. The successful electrical and Tele-data communications contractor will follow appropriate installation guidelines, as contained in the most currently available BICSI TDMM, ANSI/TIA/EIA, and NFPA 70 & 72 manuals. Additionally, contractor will work with IT Networking and Telecommunications to ensure proper placement and routing of cable and support hardware.

TELE-DATA COMMUNICATIONS CONTRACTOR QUALIFICATIONS
a. Hidalgo County requires only qualified, experienced Tele-data communications contractors perform design, project management and installation services in the construction or remodel of County telecommunications infrastructure. Pursuant to this, Hidalgo County wants to ensure that successful contractors have the capabilities, qualifications, financial stability, and experience to complete Tele-data communications installations using common industry practices (i.e. BICSI TDMM, ANSI/TIA/EIA, NFPA, etc) while meeting all Hidalgo County guidelines.

b. A consultant or contractor, by responding to a proposal, represents that their company possesses the qualifications, certifications, capabilities, test equipment, expertise, and personnel necessary to provide an efficient and successful installation of properly operating equipment as specified. 

II.  SPECIFICATIONS:
Copper

(Cost  per Installed Run of up to 285’ includes service loop)
                                                                                                                                                                                                                                  % Discount for Multi-runs
NON-PLENUM                        CAT 3 ( 4 Pair)                              CAT 5e                              CAT 6                     KEY CABLE (25 Pair)               same drop
	Regular New Construction (Rough/Pre-Punch)                                                                                                                                                                                  
	
	
	
	
	

	Regular (Post-Punch)
	
	
	
	
	

	Expedited
	
	
	
	
	

	Emergency
	
	
	
	
	


                                                                                                                                                                                                                                  % Discount for Multi-runs
PLENUM                                   CAT 3 ( 4 Pair)                              CAT 5e                              CAT 6                     KEY CABLE (25 Pair)               same drop
	Regular New Construction (Rough/Pre-Punch)                                                                                                                                                                                  
	
	
	
	
	

	Regular (Post-Punch)
	
	
	
	
	

	Expedited
	
	
	
	
	

	Emergency
	
	
	
	
	


Miscellaneous 

                                                               CAT 3 ( 4 Pair)                                    CAT 5e                                          CAT 6                               KEY CABLE (25 Pair)               
	3’ Patch Cable
	
	
	
	

	5’ Patch Cable
	
	
	
	

	7’ Patch Cable
	
	
	
	

	10’ Patch Cable
	
	
	
	

	Custom (by foot)
	
	
	
	

	66 Block (Cat 5e)
	
	
	
	

	66 Block  Cat 5e w/Connector
	
	
	
	

	5’ Amphenol Cable (25 Pr)
	
	
	
	

	10’ Amphenol cable (25 Pr)
	
	
	
	

	20’ Amphenol Cable (25 Pr)
	
	
	
	

	24 Port Patch Panel (Modular)
	
	
	
	

	24 Port Patch Panel
	
	
	
	

	48 Port Patch Panel
	
	
	
	

	110 Wiring Blocks (100 Pr)
	
	
	
	

	Cross Connect Wire 24 AGW
	
	
	
	

	Neat Patch Cable Mgr w/patch cable (Cat 6 - 48 each) Kit
	
	
	
	


AUDIO, VIDEO & ALARM

CABLE TYPE            Installed (P)            Installed (NP)        Cable Only (P)         Cable Only (NP)      With Connector (P)    With Connectors (NP)          OTHER
	RG-6 Coaxial Cable
	
	
	
	
	
	
	

	RG-59 Coaxial Cable
	
	
	
	
	
	
	

	RG-59/18-2 Combo Cable
	
	
	
	
	
	
	

	Digital Coaxial Cable
	
	
	
	
	
	
	

	18/2 Speaker Cable
	
	
	
	
	
	
	

	18/2 Security Cable
	
	
	
	
	
	
	

	22/4 Security Cable
	
	
	
	
	
	
	

	Other
	
	
	
	
	
	
	


(P-Plenum) (NP-Non Plenum)     CABLE ONLY (1,000’rolls)
FIBER OPTIC CABLE  (Multi-Mode)

REGULAR NEW CONSTRUCTION (Rough/Pre-Punch)

FIBER TYPE            NON-PLENUM          PLENUM                       OSP                      AERIAL                   ARMORDED     TERMINATION       INNER DUCT (P)         INNER DUCT (NP)

	6 Strand
	
	
	
	
	
	
	
	

	12 Strand
	
	
	
	
	
	
	
	

	24 Strand
	
	
	
	
	
	
	
	


REGULAR 

FIBER TYPE            NON-PLENUM          PLENUM                       OSP                      AERIAL                   ARMORDED     TERMINATION       INNER DUCT (P)         INNER DUCT (NP)

	6 Strand
	
	
	
	
	
	
	
	

	12 Strand
	
	
	
	
	
	
	
	

	24 Strand
	
	
	
	
	
	
	
	


EXPEDITED

FIBER TYPE            NON-PLENUM          PLENUM                       OSP                      AERIAL                   ARMORDED     TERMINATION       INNER DUCT (P)         INNER DUCT (NP)

	6 Strand
	
	
	
	
	
	
	
	

	12 Strand
	
	
	
	
	
	
	
	

	24 Strand
	
	
	
	
	
	
	
	


EMERGENCY

FIBER TYPE            NON-PLENUM          PLENUM                       OSP                      AERIAL                   ARMORDED     TERMINATION       INNER DUCT (P)         INNER DUCT (NP)

	6 Strand
	
	
	
	
	
	
	
	

	12 Strand
	
	
	
	
	
	
	
	

	24 Strand
	
	
	
	
	
	
	
	


FIBER OPTIC PATCH CABLES

DUPLEX                                      ST-ST                                   ST-SC                                     SC-SC                                        ST-LC                               LC-LC
	.3 mmPatch
	
	
	
	
	

	1m Patch
	
	
	
	
	

	3m Patch
	
	
	
	
	


FIBER OPTIC CABLE  (Single-Mode)

REGULAR NEW CONSTRUCTION (Rough/Pre-Punch)

FIBER TYPE            NON-PLENUM          PLENUM                       OSP                      AERIAL                   ARMORDED     TERMINATION       INNER DUCT (P)         INNER DUCT (NP)

	6 Strand
	
	
	
	
	
	
	
	

	12 Strand
	
	
	
	
	
	
	
	

	24 Strand
	
	
	
	
	
	
	
	


REGULAR 

FIBER TYPE            NON-PLENUM          PLENUM                       OSP                      AERIAL                   ARMORDED     TERMINATION       INNER DUCT (P)         INNER DUCT (NP)

	6 Strand
	
	
	
	
	
	
	
	

	12 Strand
	
	
	
	
	
	
	
	

	24 Strand
	
	
	
	
	
	
	
	


EXPEDITED

FIBER TYPE            NON-PLENUM          PLENUM                       OSP                      AERIAL                   ARMORDED     TERMINATION       INNER DUCT (P)         INNER DUCT (NP)

	6 Strand
	
	
	
	
	
	
	
	

	12 Strand
	
	
	
	
	
	
	
	

	24 Strand
	
	
	
	
	
	
	
	


EMERGENCY

FIBER TYPE            NON-PLENUM          PLENUM                       OSP                      AERIAL                   ARMORDED     TERMINATION       INNER DUCT (P)         INNER DUCT (NP)

	6 Strand
	
	
	
	
	
	
	
	

	12 Strand
	
	
	
	
	
	
	
	

	24 Strand
	
	
	
	
	
	
	
	


FIBER OPTIC PATCH CABLES

DUPLEX                                      ST-ST                                   ST-SC                                     SC-SC                                        ST-LC                               LC-LC
	.3 mmPatch
	
	
	
	
	

	1m Patch
	
	
	
	
	

	3m Patch
	
	
	
	
	


FIBER OPTIC MISC

                                                                                                                                                                ENCLOSURE              ENCLOSURE
                                             6-PAK (SM)                 6 PAK (MM)                 12 PAK (SM)              12 PAK (MM)              WALL MOUNT          RACK MOUNT        

	ST
	
	
	
	
	
	

	SC
	
	
	
	
	

	LC
	
	
	
	
	


WALL MOUNT RACKS 18” DEEP                24”                                                               36”                                                               48”
	Open Wall Mount Rack 
	
	
	

	Enclosed/Locking Wall Mount Rack
	
	
	

	Labor to Install
	
	
	

	
	
	
	


WALL MOUNT RACKS 24” DEEP                24”                                                               36”                                                               48”
	Open Wall Mount Rack 
	
	
	

	Enclosed/Locking Wall Mount Rack
	
	
	

	Labor to Install
	
	
	

	
	
	
	


FLOOR MOUNT RACKS

DESCRIPTION                                                                                                                                                         7’                                                  Other
	Floor Mount Open Rack Alminum
	
	

	Installation
	
	

	Floor Mount Open Rack  4 –Post Frame (Black)
	
	

	Installation
	
	

	Open Cabinet with Casters (Black)
	
	

	Installation
	
	

	Open Cabinet with Side Panels Only (Black)
	
	

	Installation
	
	

	Cabinet with Side Panels and Front and Back Locking Doors
	
	

	Installation
	
	

	Ladder Rack Assembly 10’ Sections & Hardware
	
	

	Installation
	
	

	Vertical Cable Management for Open Floor Mount Rack
	
	

	Fan/Filter Kit for Wall-Mount Rack with Enclosure
	
	

	Fan/Filter Kit for Enclosed Floor Mount Cabinet
	
	


UPS

Description                                                                      Tower                                                       Rack Mount                                                   Other
	500
	
	
	

	750
	
	
	

	1000
	
	
	

	1200
	
	
	

	1500
	
	
	

	1750
	
	
	

	2000
	
	
	

	3000
	
	
	


MISCELLANEOUS

      DESCRIPTION                              PRICE PER UNIT                                                                         DESCRIPTION                                  PRICE PER UNIT
	J HOOKS Wall Hangers
	
	
	Concrete Penetrations
	

	Cable Support Saddles
	
	
	Fire Wall Penetrations
	

	D-Rings
	
	
	Asbestos Penetration
	

	Wire Mold Raceway
	
	
	
	

	Wire Mold Boxes
	
	
	
	

	Wire Mold Clips
	
	
	
	

	Low Voltage Single Gang Box
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	

	
	
	
	
	


INFRASTRUCTURE  FOR COOPER & FIBER EXTERIOR WORK  per foot

                     2” CONDUIT    4” CONDUIT    TRENCHING  TRENCHING    4” BORE             6” BORE         PULL BOX       PULL BOX

                                                                                                          (ASHPALT/CALICHE)                                                                                   2X3X2                       6X4X3
	Regular
	
	
	
	
	
	
	
	
	

	Expedited
	
	
	
	
	
	
	
	
	

	Emergency
	
	
	
	
	
	
	
	
	

	Long Bend Elbows
	
	
	

	Regular Elbows
	
	
	

	LB ACCESS FITTINGS
	
	
	


Trenching 3’ deep by 4” wide
SERVICE CALL/TROUBLE SHOOTING
Price Per Hour

                                            FIBER OPTIC                    DATA                TELEPHONE/FAX    ACCESS CONTROL      AUDIO/VIDEO                   OTHER    

	Regular Hours
	
	
	
	
	
	

	After Hours/Holiday
	
	
	
	
	
	

	Expedited
	
	
	
	
	
	

	Emergency
	
	
	
	
	
	


ETHERNET-ENABLE CONTROLLERS

(Power over Ethernet)

Description                                               Price Per Unit                   Installation & Programming

	INTEGRAL Intelli-m 4 Control Module Kit
	
	
	
	

	INTEGRAL Intelli-m Single Control Module Kit
	
	
	
	

	INTEGRAL Single Module Add-on Kit
	
	
	
	

	Proximity Reader  
	
	
	
	

	Electric Door Strike  (Wood)
	
	
	
	

	Electric Door Strike (Metal) 
	
	
	
	

	Electromagnetic Lock
	
	
	
	

	Request to Exit Buttons
	
	
	
	

	12/24 Power Supply
	
	
	
	


III. 
SPECIFICATIONS:

1. Except as indicated by Hidalgo County IT Telecom Services, Contractor shall provide materials and labor for cable installation work throughout Hidalgo County; to include new work as well as repairs to existing. Completed jobs shall conform to the Hidalgo County Voice and Data Wiring Specification Manual.
2. Contractor shall prepare a written job cost estimate for each job based upon a visit to the job site. Contractor must be available to visit job sites based on the following classifications of notification:
(a)
Routine – five (5) working days of notification
(b) Expedite – one (1) working day of notification

(c) Emergency – two (2) hours of notification

3. Contractor shall provide an electronic copy of the completed request for estimate form, (see Exhibit B, page of this bid), to the ITTS Department based on the following classifications of notification:

(a) Routine – within forty-eight (48) hours after visiting the site.

(b) Expedite – within twenty-four (24) hours after visiting the site.

(c) Emergency – verbal estimate to be provided at the time of site visit with written follow-up of verbal estimate within twenty-four (24) hours of visit.
4. Once Hidalgo County has accepted a cabling job estimate through the issuance of a valid purchase order, the dollar amount reflected on the purchase order shall constitute the County’s total obligation for that job. The Contractor shall endeavor to submit an accurate cost estimate. No claims for compensation will be considered or allowed for extra work resulting from ignorance of any existing conditions on the part of the proposer. However, the County recognizes that some elements of work cannot be detected until work is in progress. When latent physical conditions are discovered, the Contractor shall immediately notify the IT Telecom Manager, and before work continues, the Contractor shall submit a revised cost estimate for approval. A written change order to the existing purchase order or the issuance of an additional purchase order is the only valid approval mechanisms for additional work. 

5. Contractor shall inform the IT Department of the work in progress and advise them when a job is complete. For long-term projects (those exceeding five (5) working days), the Contractor shall meet with ITTS representative and provide a weekly, typewritten progress report. This report should be electronically submitted no later than 5:00 p.m. on each Friday until the work is completed. Weekly meetings will include a site inspection to ensure compliance with established standards.
6. Permits, Fees and Notices:
a. The Contractor shall secure and pay for the building permit and for all required permits, governmental fees, licenses, and inspections necessary for the proper execution and completion of the work. Contractor’s estimate should include cost of any permits, fees, or notices for each job.
b. The Contractor shall give all notices and comply with all laws, ordinances, rules, regulations, and lawful orders of any public authority bearing on the performance of the work.
c. If the Contractor performs any work when he knows or should know it to be contrary to such laws, ordinances, rules and regulations, and shall therefore bear all costs attributable thereto.

7. Selected Contractor shall be expected to meet established schedules and deadlines for the completion of work as outlined in job estimates.

8. Selected Contractor shall be expected to adhere to current Hidalgo County Voice and Data Wiring Specification Manual. It is the responsibility of the Contractor to request any new revisions to the wiring specifications at the time an estimate is provided.

9. Proposer should have a current BICSI certified Registered Communication Distribution Designer (RCDD) on staff as a full-time employee. A copy of the RCDD certificate and BICSI member number must be provided with bidding documents
10. Proposer shall provide a representative list of three (3) or more references including contact name and telephone number. These references should represent the installation of copper and fiber optic cabling in interior, exterior, aerial, and underground environments. The Proposer should indicate which types of work have been conducted for each reference. Hidalgo County may contact these references as one means of collecting information regarding past performance of the Proposer. THIS INFORMATION SHALL BE PROVIDED WITH PROPOSAL.

11. Proposer shall provide a document detailing the staffing level of the Proposer’s firm by job classification (i.e. cable installer, installers helper and qualifications of staff). THIS INFORMATION SHALL BE PROVIDED WITH PROPOSAL.

12. Proposer shall provide a list of equipment owned; fiber and copper, description and any additional information as needed. THIS INFORMATION SHALL BE PROVIDED WITH PROPOSAL.

13. Proposer shall provide a list of certificates and cables covered by manufacturer training and certification. THIS INFORMATION SHALL BE PROVIDED WITH PROPOSAL.

14. Proposer shall warranty its materials, design, and workmanship for a minimum of one (1) year from the date of work completion and shall include all parts, labor, and transportation as specified.

15. Warranty information on installed equipment and cabling shall be detailed in each job estimate. Proposer shall warranty its materials, design, and workmanship for a minimum of one (1) year from the date of work completion and shall include all parts, labor, and transportation.

16. Warranty shall commence on each job on the date it is accepted as complete by Hidalgo County.

17. The Contractor shall respond with remedy to a warranty trouble call within twenty-four (24) hours after receipt of such a call.

18. If it becomes necessary for the County to contact another vendor for warranty repairs due to inability or failure of the Contractor to perform such repairs, the Contractor shall reimburse the County for all invoices for labor, materials required and the shipping/handling costs thereof, to perform such repairs, within thirty (30) calendar days from presentation of such invoice. This shall only occur after the Contractor has been given reasonable time and fair opportunity to respond and correct the problem. The cost limitation for such repairs will not exceed the parts and labor replacement price of the repair.

19. All cable/equipment installed must be fully tested and meet TIA/EIA testing and installation standards for UTP/STP copper, coaxial, and SM/MM fiber optic cable. This includes certification for Cat5, Cat3, Cat6 installation, Fast Ethernet, (100MB), Gigabit Ethernet (1000 MB), FDDI, etc. All test results shall be submitted electronically to designated Hidalgo County ITTS representative within five (5) days of test completion. Invoices will NOT be paid unless corresponding test results are received.
IV.  Terms & Conditions:

1. SCOPE of CONTRACT: This is a firm fixed-price, estimated quantity contract calling for delivery of the products identified in Description at the stated prices submitted by the Proposer.  Upon acceptance of a bid by Hidalgo County Commissioners and issuance of a Contract Award by the Hidalgo County Purchasing Agent, Proposer shall be obligated to deliver the products at the stated prices, within the time specified, and in accordance with all Terms and Conditions and Description contained herein.  Delivery schedule listed by Proposers on the face of this RFB shall apply to the individual Purchase Order. Hidalgo County will seek purchases from State-awarded contracted vendors, cooperative purchasing programs, or any other vendor whenever it is statutorily permitted and in its best interest to do so.  Hidalgo County is not obligated to order products not covered by this contract and is free to secure those requirements from other sources in accordance with applicable purchasing statues. Hidalgo County will seek purchases from State-awarded contracted vendors, cooperative purchasing programs, or any other vendor whenever it is statutorily permitted and in its best interest to do so.
2. TERM CONTRACT: The initial term of this contract shall commence upon award for twelve (12) months with Hidalgo County Commissioners Court sole discretion to extend for one (1) additional year, subject to approval by the Commissioners Court under the same firm fixed prices, terms and conditions.
3. PRICE CHANGES: The prices offered shall remain firm for the period of the contract.  The prices offered shall also remain firm for the option year should the County choose to exercise the option to renew, except for changes that are industry wide and beyond the control of the contractor.  If such changes do occur, it will be the responsibility of the contractor to provide documentation to Hidalgo County substantiating the changes to the bid process.  Any price changes must be approved by Hidalgo County.

1. METHOD OF AWARD: Award, if any, will be made to 

a. Primary Vendor the responsible Proposer(s) submitting the lowest bid prices(s) and meeting all requirements of this Request for Proposal. 

b. Secondary Vendor the responsible Proposer(s) submitting the second lowest proposal(s) and meeting all requirements of this Request for Proposal.

Hidalgo County reserves the right to award on an “all or none” or “by line item” basis.  Proposer may propose on any or all items in order to be considered responsive to this Request for Proposal.
4. PURCHASE ORDER: Contractor will not supply and/or deliver any items until a purchase order number is assigned by the designated representative of the County Purchasing Office.  Contractor will reference purchase order and contract number on all invoices submitted to the Hidalgo County Auditor.  The Contractor must respond by supplying and/or delivering the parts.  Failure to act in this manner may result in termination of this contract.
5. MATERIAL COST:  Prices for each job shall include the cost of shipping and delivery of all items to the designated FOB point and shall not include Federal or State of Texas sales, excise and use taxes.  Proposer must include all costs in the delivered unit price.
6. REQUIRED DELIVERY:  Verbal Purchase Order’s placed by 3:00 p.m. shall be delivered to the department that same business day.  Order’s after 3:00 p.m. shall be next business day deliver.

7. DAMAGES:  Contractor will be totally responsible for any and all damages and/or losses to equipment and property which may occur during the performance of this contract.  Damages to equipment and property will be assessed by an agent of the Hidalgo County Elections Department.

V. ADDITIONAL INFORMATION:
1. All costs and expenses associated with preparation and submission of (bids, proposals and/or quotes) shall be the responsibility of the Proposer and no reimbursements for such charges or expenses shall be passed onto Hidalgo County. 

2. Hidalgo County is requesting that any and all questions, inquiries and clarifications regarding quotes, bids, proposals or statements of qualifications be addressed to:  Martha L. Salazar, CPPB, Purchasing Agent, Hidalgo County Purchasing Department, 2812 S. Business Hwy. 281, New Administration Building, Edinburg, Texas 78539.  TELEPHONE INQUIRIES WILL NOT BE ACCEPTED.
3. ALL WRITTEN INQUIRIES WILL BE ACCEPTED VIA FACSIMILE NO LATER THAN, Wednesday, ______ by 5:00 P.M... Responses to said inquiries will be sent to all applicants via facsimile by no later than, Friday, ______ by 5:00 p.m..
PAGE  
9

