PAVEMENT CUT REPAIR

1 PERMITTEE SHALL PROVIDE ADEQUATE TRAFFIC CONTROL SIGNS AND PERSONNEL AS REQUIRED BY THE TEXAS MANUAL OF UNIFORM TRAFFIC CONTROL DEVICES.

2 LINE AND GRADE SHALL BE STAKED BY PERMITTEE 24 HOURS IN ADVANCE OF CONSTRUCTION FOR COUNTY INSPECTION OF OTHER AFFECTED UTILITIES. 

3 ALL BACKFILL SHALL BE PERFORMED IN 8 INCH LOOSE LAYERS COMPACTED TO 95% STANDARD PROCTOR DENSITY. 3% CEMENT STABILIZED BACKFILL SHALL BE PLACED FROM 18 INCHES BELOW FINISHED GRADE TO TOP OF SUBGRADE.  TESTING OF BACKFILL MATERIAL AND COMPACTION SHALL BE THE RESPONSIBILITY OF THE PERMITTEE. TESTING RESULTS SHALL BE PROVIDED TO THE HIDALGO COUNTY PLANNING DEPARTMENT AND/OR HIDALGO COUNTY ROW DEPARTMENT BEFORE THE COVERING OF THE TRENCH. IF COMPACTION FAILS. THEN PERMITTEE SHALL REMOVE BACKFILL MATERIAL AND REINSTALL IN SAID 8 INCH LIFTS AND RE-COMPACT TO MEET SAID COMPACTION SPECIFICATIONS. 

4 THE PAVING STRUCTURE SHALL BE REPLACED WITH A 2 INCH HMAC AND 8 INCH BASE. 

5 ALL PAVING CUTS SHALL BE IN STRAIGHT LINES AND THE EDGES EITHER CUT OR BROKEN IN UNIFORM SQUARE EDGES. 

6 THE PATCH ON ASPHALT STREETS SHALL EXTEND A MINIMUM OF 36 INCHES ON EITHER SIDE OF THE TRENCH CONCRETE STREETS SHALL HAVE AN EXPANSION JOINT WHERE NEW CONCRETE ABUTS OLD. 

7 ANY FUTURE SETTLEMENT SHALL BE REPAIRED BY THE PERMITTEE FOR A PERIOD OF ONE YEAR. 

8 ANY SHRUBBERY, DOMESTIC PLANTS OR GRASS REMOVED DURING CONSTRUCTION SHALL BE REPLANTED AND MAINTAINED BY QUALIFIED PERSONNEL FOR 3 MONTHS. 

9 ANY EXISTING SIGNS SHALL BE REMOVED AND REINSTALLED BY THE PERMITTEE ONLY UNDER THE SUPERVISION OF THE HIDALGO COUNTY ROW AND/OR THE PLANNING DEPARTMENT. 

10 THE ISSUANCE OF THE PERMIT GRANTS PERMISSION TO WORK WITHIN COUNTY ROW FOR THE PURPOSE OF LAYING THE PROPOSED FACILITY, BUT DOES NOT GUARANTEE A ROUTE FREE OF OBSTRUCTIONS SUCH AS UTILITY LINES WHETHER PRIVATELY OR COMMERCIALLY OWNED IN ORDER TO PREVENT DAMAGE TO THESE UTILITY LINES IT IS THE PERMITTEE'S RESPONSIBILITY TO CONTACT THE VARIOUS UTILITY COMPANIES OR PRIVATE OWNERS FOR THE EXACT LOCATIONS OF ANY FACILITIES THAT MAY BE IN THE PATH OF THE PROPOSED WORK. 

11 THE ROW MUST BE CLEARED OF TRASH AND EXCESS DIRT AND LEFT IN A NEAT, CLEAN CONDITION UPON COMPLETION OF THE INSTALLATION. 

12 IN CROSSING, THE TOP OF THE UTILITY LINE SHALL BE A MINIMUM OF 2 FEET BELOW GRADE LINE IN DITCHES OR A MINIMUM OF 4 FEET BELOW THE PAVEMENT EDGE. WHICHEVER IS GREATER. IN CURB AND GUTTER SECTION YOUR LINE SHALL BE COVERED A MINIMUM OF 2.5 FEET BELOW THE FLOW LINE OF THE GUTTER WHERE THE BORING METHOD IS REQUIRED BY THE COUNTY, OVER CUTTING IN EXCESS OF 1 INCH DIAMETER SHALL BE REMEDIED BY PRESSURE GROUTING THE ENTIRE INSTALLATION.

[image: image1.png]f——VARIABLE —

INSIDE DIA. OF
PIPE PLUS 127

HAND TAMPED
ACCORDING TO SPECS.

i

" MINg

TRENCH TO BE EXCAVATED  ©

TO CONFORM TO LOWER QUARTER
OF OUTSIDE OF PIPE GRAVEL BEDDING
WASHED GRAVEL IN ACCORDANCE WITH
TXDOT SPECIFICATIONS





[image: image2.png]~—VARIABLE —

INSIDE DIA. OF
PIPE PLUS 127

HAND TAMPED
ACCORDING TO SPECS.

CONCRETE CRADLE




