December 8, 2010

Legacy Christian Academy

Attn: David Anderson

3037 Bunker Lake Blvd NW

Andover, MN 55304

Re:  	Site Plan & Plat Review

Alpha Development

Legacy Christian Academy

Mr. David Anderson:

We are in receipt of your Plat for a subdivision to create one buildable lot and four outlots. The final plat includes two sheets prepared by Anderson Engineering, dated November 4, 2010.  We are also in receipt of the Site Plan submittal consisting of thirteen (14) sheets prepared by Cuningham Group and Anderson Engineering.  The sheets are identified as follows: Sheets A.1, A.2, A.3, A.4, Dated and C1, C2, C3, C4, C5, C6, L1 Dated November 1, 2010 and E-1 and E-2 (Parsons).  We offer the following comments regarding your site plan and plat submittal:

Planning and Zoning

Reviewer: Amber G. Miller, Planning Manager

amiller@ci.ramsey.mn.us

763-433-9843

General:  The submitted major subdivision proposes to plat approximately 184.97 acres (including portions of Pine Shadows Addition and currently unplatted lands) to create one buildable lot (approximately 86.436 acres), four outlots (35 acres, 45.196 acres, 4.175 acres and 9.621 acres) and right-of-way (8.64 acres). The subject property is generally located west of Armstrong Blvd, south of Alpine Dr and north of the railroad tracks.

Zoning:  The subject property has multiple zonings including R-1 Residential, R-2 Residential, R-3 Residential and TC-2 (Town Center).  Schools, whether private or public, are not a permitted use in any of these districts.  We are in receipt of a zoning amendment application to rezone the property to Public/Quasi Public, which does include both public and private schools as a permitted use.  As has been past practice for the Public/Quasi-public zoning district, this review letter will evaluate the proposal based on the standards of the B-1 Business District or the most appropriate zoning district based on bulk standards.

Lot Size Requirements:  The minimum required lot area is ½ acre and the minimum required lot width is 100 feet.  Lot 1 is proposed to be 86.436 acres which are proposed to rezoned to Public/Quasi Public and require Comprehensive Plan Amendment; while Outlots A and B are proposed to be 36.79 acres and 45.24 acres, respectively. Outlot C, the lot being dedicated to the City, is 4.175 acres.  Outlot D, immediately adjacent to Armstrong is 9.621 acres.  Additionally, each lot shall have a minimum depth of 150 feet.  All four lots conform to the minimum lot size requirements.  Additionally, there will be 8.84 acres of Right-of-Way.

Architectural Standards:  Some combination of brick, stone, stucco, wood, glass would be required for the façade of the building.  All other materials would have to be approved by the City Council.  The material use of the dome structure will have to be approved by the City Council.

Building Height:  The ‘main’ portion of the roof is just under 32 feet in height, various portions of the building are shown to be at 38 and 40 feet in height (gym and classroom roofs, which are flat roofs).  The temporary dome structure must be less than 65 feet in order be compliant with City code and the applicant has indicated that it will comply.

Access and Streets:  The site as submitted proposes two accesses from an extension of Bunker Lake Blvd. as a local street within a proposed 120 foot right-of-way. Upon development, Outlot A would access an improved Puma St. and Alpine Dr. and Outlot B would access from the extended Bunker Lake Blvd. and potentially an improved Puma St.  Full build out of Bunker Lake Blvd it is planned as a four lane road with two lanes in each direction.  A first phase is being proposed to construct one lane in each direction, cul-de-sac terminating just west of the softball fields, and a trail on the north side of Bunker Lake Blvd to Puma St and then north to Outlot A.  The proposed cul-de-sac exceeds the 600 foot maximum length that exists in Section 117-614(c)(7) of City Code, therefore revisions should be made to extend Bunker Lake Boulevard all the way to Puma Street.  Puma Street should then be upgraded to minimum City paving standards to Alpine Drive (including a trail) to serve as a secondary access into the project area.

Consistent with current negotiations with the applicant the phase 1 plan has been revised to construct Bunker lake Boulevard one lane in each direction with a trail along the north side, all utilities, and curb and gutter up to the second entrance to the school, which is approximately 2,700 feet.  The remaining 1,250 feet to Puma Street would be constructed with a reduced section of no utilities, no curb and gutter, and a twenty four foot wide section of asphalt with 6 inches of class 5 base.  Puma Street would then be upgraded to minimum City paving standards to Alpine Drive (including a trail on the west side) to serve as a secondary access into the project area.  In addition, the westerly leg of the Bunker Lake and Armstrong Blvd intersection will be constructed full width.  The ultimate infrastructure improvements required by the City to serve the project will be determined by the City Council at a Public hearing on the feasibility study scheduled as a separate item on this Council agenda (December 14, 2010). The applicant will be required to enter into an interim agreement for engineering services prior to the development agreement to ensure payment for the design services for the agreed upon infrastructure improvements from the feasibility study.

Consistent with the City’s comprehensive trail plan and 2030 Comprehensive Plan, an off road trail easement is shown on the northern edge of the wetland on Lot 1 and the southern edge of Outlot A at 20 feet in width.  Further, additional trail easements would be required along the east side of Lot 1 so that a continuous route is maintained through the site from Puma/Alpine to Bunker/Armstrong.

Landscape Review

Reviewer: Chris Anderson

canderson@ci.ramsey.mn.us

763-433-9905

Landscape Plan:  Based on the lineal footage of the site (11,550 feet), the number of proposed parking stalls (692) and the additional required plantings for the bufferyard area, a total of 358 trees and 385 shrubs are required.  The proposed landscaping plan includes 320 newly planted trees (as well as about thirty-four [34] existing trees that will be preserved) and includes a list of various shrub species that will also be planted (but without identifying quantities).  The following comments are offered regarding the revised Landscaping Plan, dated November 1, 2010, prepared by Anderson Engineering:

- The landscape plan must indicate quantities of each species and exact location of each species throughout site (by way of unique symbols or labels).

- Minimum required size at time of planting is twenty-four (24) inches in height/width for shrubs, whichever dimension is more applicable per species.  Minimum size must be included on the plan.

- At each of the four (4) entrances to the site, the vision clearance triangle must remain free of any plantings to ensure unobstructed site lines.  The vision clearance triangle can be determined by connecting two (2) twenty-five (25) foot ‘legs’, which run parallel with the curb, with a third leg.  This area shall remain free of any plantings and should be indentified on the landscaping plan.

- The landscaping plan must differentiate those areas that are to have sod and those areas proposed to be seeded.  City Code requires all disturbed areas to be sodded.  Any deviation from this condition requires City Council approval.

- All landscaped areas must include in-ground irrigation.  The irrigation system must include rain sensors and back-flow prevention devices, which must be noted on the plan.

- All irrigated and landscaped areas must be covered with six (6) inches of topsoil that meets MnDOT’s Premium Topsoil Borrow specification.  If amending onsite topsoil, documentation must be supplied to the City to verify that the amended material has been tested and meets MnDOT’s Premium Topsoil Borrow specification. The topsoil requirement and specification must be noted on the plan.

- A forty (40) foot bufferyard between the school and the adjacent single family lot on Puma St must be maintained and should include at least fifty-eight (58) trees (which is equivalent to an additional 25% of the required site plantings [231 trees excluding the parking lot plantings]).  The buffer width on the plan is acceptable but additional plantings are required (while some landscaping exists on the single family lot off Puma St, it is the responsibility of the Applicant to provide all required plantings and area to satisfy the buffer yard requirement).

  - Additional plantings should consist of coniferous species to provide more year round buffering.

  - Spacing between plantings should be reduced.  It appears that the trees are about thirty-five feet on center.  This distance can be reduced by almost fifty percent (50%) to provide more thorough screening/buffering.

  - Incorporating multiple rows of trees, at staggered positions, is also encouraged to provide more thorough screening/buffering.

  - Note that the buffer yard requirements could be satisfied by repositioning some of the proposed site landscaping.

Tree Preservation Plan: A tree preservation plan, prepared by Anderson Engineering and dated September 3, 2010, has been submitted.  Tree protection fencing must be installed prior to the commencement of any earthwork.  There shall be no trenching within the dripline of protected trees nor shall there be any storage of equipment, materials, debris, soil deposits or fill within the protected tree zone.

Engineering Review

Reviewer: Tim Himmer, City Engineer

thimmer@ci.ramsey.mn.us

763-433-9893

Traffic Analysis:  A traffic analysis was submitted for review, and it generally conforms to the existing traffic study that was prepared for the previous residential development on this site (SUMMERSET MEADOWS)  The analysis confirms that traffic volumes from this proposed development will be less than those originally proposed under the SUMMERSET MEADOWS proposal, so no additional modifications will be required to the planned intersection improvements at Armstrong Boulevard and Bunker Lake Boulevard.    Additional traffic analysis may be required when the proposed Outlots advance to the development stage and a project has been submitted for review.  Anoka County has also been forwarded this analysis for review and comments.

Grading and Drainage: The proposed grading plan appears to work generally well, but there are a few locations that should be evaluated for improvements.  All internal access roads and parking areas must have concrete curb and gutter on the edges and around the perimeter.  The north slope for Pond-1 should be reinforced to protect against erosion concerns from the proposed sheet flows to the pond in this location.  Attempts should be made to relocate pond inlets and outlets such that greater separation exists for adequate stormwater treatment; this concern is most prevalent within ponds 1 and 5.

Based upon the improvements proposed permits will be required from the Lower Rum River Water Management Organization and the MPCA (NPDES construction permit).Revised drainage calculations have been received by City staff, and this information was also submitted to the LRRWMO for review as part of the watershed permit requirement for this project.  Staff will defer to the LRRWMO as it relates to the proposed private stormwater system that will not discharge to the existing wetland complex on the property (not to the public system), and the proposed stormwater system for the public infrastructure portions of the project will be reviewed as part of the final design for the improvements proposed under the feasibility study.  Additional items that will be evaluated within this feasibility study include; pipe material and sizing, utility extensions, roadway design, cost breakdowns, trails, etc.  Maintenance agreements will be required for all private storm ponds to ensure that they continue to function as designed and will remain in perpetuity.

Is there any dewatering required for this project?  If so, the appropriate permit must be secured from the DNR, and the City will want to review the dewatering plan prior to implementation.  No dewatering activities can occur until the required permit is secured and submitted for review.

Notes on the grading plan must be added and/or revised to include the following:

- Removal of note #18; no concrete wash out areas are allowed, consistent with revised rules from the MPCA on this matter.

- Note #9 must be revised to include language that any sediment tracked onto public streets must be cleaned up immediately upon discovery, or as directed by the City Engineer.  The contractor must have a sweeper on site or available upon 3 hours notice.

- Installation of silt fence an all other down gradient sediment protection measures shall be completed and inspected prior to commencement of any site activities.

Easements:  Standard drainage and utility easements ten (10) feet in width abutting dedicated right-of-way and five (5) feet in width along both side and rear lot lines are shown on the plat.  Additional drainage and utility easements are required to encumber the 100-year flood zone for all existing and proposed surface waters and wetlands, consistent with negotiations for reduced developments fees on a net developable area calculation.  Easements must also be dedicated over all utility trunk lines that extend internal to the site so the City has access to maintain those facilities.

Right-of-Way Dedication: Dedications are proposed to maintain a continuous one hundred and twenty foot (120’) right-of-way for the Bunker Lake Boulevard extension.  Forty feet (40’) of half right-of-way must also be dedicated along the entire length of Puma Street, consistent with the PINE SHADOWS subdivision, through the extent of this proposed plat.  Additionally, the right-of-way dedication adjacent to Outlot A for Alpine Drive must be expanded to 40’ in width.

Utilities: Utility plans have been submitted for review, and those plans will be evaluated through the requested feasibility study process.  Water service should be added along Bunker Lake Boulevard to Outlot B, similar to the proposed storm and sanitary sewer stubs, to serve that property.  Based upon negotiations it will be the applicant’s responsibility to update all plans (site, street, utility, plat, etc.) consistent with the newly proposed phase 1 construction schedule, as determined by the City Council at the public hearing on the feasibility study for the infrastructure improvements to serve this project.  The costs associated with such extensions to serve this project will be paid by the developer, with any oversizing necessary to serve the larger service district being paid by the City’s corresponding Utility Funds.

All watermain 6” in diameter and smaller shall be class 53 DIP, and all watermain greater than 6” in diameter shall be class 52 DIP.  All storm sewer connections to the public system must be done with RCP; HDPE pipe material is only allowed as part of the private system.

Future phase development of Outlot B and the southerly 2 lanes of Bunker Lake Boulevard, when constructed, must make accommodations for a future raw water line that would serve the City surface water treatment plant; this will be evaluated when such improvements are triggered by future development.

Park & Trail Fees:  The 2010 rate for Park Dedication and Trail Fees are $4,738/acre and $1,090/acre respectively for commercial properties.  Because City staff and the applicant continue to dialogue about the exact configuration of lot lines and sizes, this fee will be determined at the time of final plat.  At the City Council Meeting October 26, 2010, the Council accepted the recommendation of the Park Commission of a 4.175 acre Outlot.   This fulfills $395,623 in Park Dedication (Value afforded to LCA = $94,760 per acre).  Trail fee is estimated to be $70,043.  We understand you are requesting a waiver of the City’s trail development fee in lieu of construction of trails along the extension of Bunker Lake Boulevard and the west side of Puma Street.  City Code only allows for credit for portions of trails constructed off-site, which equate to a credit of approximately $12,000.  Any deviation from this credit standard shall require City Council approval.

Sanitary Sewer, Water, Stormwater Trunk Fees.  The 2010 rate for Sewer, Water, and Storm-water Trunk Fees are $3,824/acre, $8,337/acre, and $4,465/acre respectively for commercial properties.  Because City staff and the applicant continue to dialogue about the exact configuration of lot lines and sizes, this fee will be determined at the time of recording of final plat.  Based on the reduced rate for net area instead of gross area it would equal $245,730 for Sewer, $535,736 for Water and $250,040 for Stormwater.  The final amount collected will be dependent upon credits available based on the final construction costs determined through the assessment project request.

Lighting: City standards require that the luminaire (light source) must not be visible from beyond the property line.  Any lighting proposed to illuminate off-street parking areas, signs, or facilities/structures shall be arranged to deflect light away from Bunker Lake Blvd and adjacent properties.  No lighting is proposed along the westerly extension of Bunker Lake Boulevard at this time, but should Council choose boulevard lighting could be included in the final design drawings for that project.  If added, such lighting would comply with the City’s priority streetlighting policy.  An internal site lighting plan must be submitted for review, including the parking areas and any proposed athletic field lighting.  The site lighting plan should include lights placed at each entrance to adequately light the access points for safe maneuverability.

REU Number: This property will be subject to a storm water utility charge based upon the amount of impervious surface on the site.  The quarterly charge is dependent upon the number of Residential Equivalent Units (REU) calculated for this property.  Please provide the overall property size, and impervious area calculations to assist in determining this fee.

If you have any questions, please call our office at (763) 427-1410

The City Council will be meeting at 7:00 p.m. on Tuesday, December 14, 2010 to review your final plat and site plan.  You, or your representative, are strongly encouraged to attend the meeting in order to answer any questions that may arise. If you have any questions or need additional information, please don’t hesitate to call us.

Respectfully,

# CITY OF RAMSEY

Chris Anderson				Amber Miller			Tim Himmer

Environmental Coordinator		Planning Manager		City Engineer

CC: 	Anderson Engineering of Minnesota, LLC

Attn: Troy Gamble

13605 1st Avenue North

Plymouth, MN 55441

Cuningham Group Architecture, P.A.

Attn: Brian Tempas, AIA

201 Main Street SE, Suite 325

Minneapolis, MN 55414